SMART HOME MOBILE APPLICATION

Consolidated Detailed Functional Specification

Cecil Amartey Abbey Project

Version 2.0 — Consolidated Edition

Merges the baseline application specification with the production-grade six-pillar architecture, plus expanded non-functional, ecosystem, and data-model requirements.

# Table of Contents

# 0. Purpose & Scope

This document consolidates two source specifications into a single, production-grade functional specification for the Smart Home Mobile Application: (1) the original room-by-room, scene-and-device-oriented functional specification prepared for the Cecil Amartey Abbey project, and (2) the six-pillar, production-grade architecture (Onboarding & Provisioning; State Management & Dashboard; Advanced Automation; Energy & Health Analytics; Security & Video Telemetry; User Management & Security).

Beyond merging the two, this edition adds material not present in either source: non-functional requirements, third-party ecosystem/interoperability requirements, voice-assistant and AI integration, accessibility requirements, backup/disaster-recovery, a starter data model, and a REST/event API sketch. New material is flagged inline where useful so the project team can trace additions.

Numbers such as latency and update targets are stated as design targets to be validated against the selected hardware and network topology during detailed design.

# 1. Onboarding & Provisioning

Governs how a new device, room, or entire property is added to the system, from unboxing to first live control.

### 1.1 Zero-Touch & Discovery

- Matter QR-code scanning for standards-based commissioning.

- Bluetooth Low Energy (BLE) beaconing for proximity-based device discovery.

- mDNS/Bonjour broadcast detection for local-network device discovery.

- NFC tap-to-pair for supported access-control and sensor hardware.

- Fallback manual entry (setup code / serial number) when automatic discovery fails.

### 1.2 Network Handshake

- Auto-transfer of Wi-Fi and Thread credentials to target nodes without manual SSID selection.

- Automatic Thread Border Router election and hand-off among capable hubs.

- Network health check (signal strength, channel congestion) before finalizing pairing.

- Graceful retry/backoff and clear error messaging on handshake failure.

### 1.3 Firmware Validation & OTA

- Mandatory firmware-version check against a minimum security baseline during provisioning.

- Automatic Over-The-Air (OTA) update trigger when a device is below the baseline.

- Staged OTA rollout (canary → percentage rollout → full fleet) for house-wide firmware pushes.

- Rollback to last-known-good firmware on failed or bricked updates.

### 1.4 Guided Setup Flow (new)

- Wizard-style room assignment during provisioning, pre-populated from the floor plan where available.

- Bulk-provisioning mode for installers commissioning many devices during a single site visit.

- Digital handover package generated at project completion: device list, firmware versions, network map, and warranty records, exportable as PDF.

- Installer-only "technician mode" with elevated diagnostics, hidden from standard homeowner accounts.

# 2. State Management & Application Dashboard

The dashboard is the homeowner's single overview of the residence; state management is the underlying real-time data layer that keeps it accurate.

### 2.1 Dashboard Content

- Current home mode: Home, Away, Night, Sleep, Vacation.

- Overall security status and any active alarms or warnings.

- Indoor and outdoor temperature; current and daily electrical power consumption.

- Door/window status, active lighting zones, HVAC operating status, water-leak status.

- Communication/system faults and offline-device count.

- Quick-access buttons for major scenes: All Lights Off, All Loads Off, Away, Night, Home, Emergency/Alarm.

### 2.2 Dynamic Room Grouping

- Hierarchical spatial organization: House → Floor → Room → Zone.

- Drag-and-drop editing of the hierarchy from the app, with changes reflected instantly in all views.

- Zone-level grouping within a room (e.g., "Reading Nook" as a lighting zone inside "Living Room").

### 2.3 Multi-Property Management

- Fast switching between primary residence, vacation rental, or commercial spaces within a single account.

- Per-property notification routing so alerts from an unoccupied vacation property are clearly distinguished.

- Independent user/role lists per property, with an option to share a single owner identity across all properties.

### 2.4 Real-Time State Mirroring

- Low-latency state sync via WebSockets or MQTT so UI toggles reflect hardware state within ≤100 ms on local network.

- Optimistic UI updates with automatic rollback if the device fails to confirm the command.

- Conflict resolution when a device is toggled simultaneously from the app, a physical switch, and a voice assistant.

### 2.5 Widget & Lock-Screen Controls

- Native OS integration: iOS Control Center widgets and Android Device Controls / Quick Settings tiles.

- Home-screen and lock-screen widgets for favorite scenes and the security-arm toggle.

- watchOS / Wear OS companion controls for scene activation (new).

# 3. Room Control

A room-by-room interface lets the user drill into any space and control everything assigned to it.

### 3.1 Per-Room Display

- Room name, lighting status, HVAC status, temperature, presence/occupancy status.

- Window status, audio status, and status of any other controlled equipment (shades, garage door, irrigation).

### 3.2 Per-Room Controls

- Select a room and control all devices assigned to it from one screen.

- Save a room-level "favorite state" that can be recalled with one tap.

Master Bedroom Example — Ceiling light ON/OFF; light dimming 0–100%; RGBW control where applicable; tunable-white control. Example — HVAC temperature setting; fan control; audio control; window status; presence status.

# 4. Lighting Control

### 4.1 ON/OFF Control

- Switch individual lights ON/OFF.

- Switch groups of lights ON/OFF.

- Switch all designated house lights OFF with a single control.

### 4.2 Dimming

- Brightness range 0–100% via slider.

- Store preferred brightness levels per light or group.

- Support fade-in/fade-out transitions.

- Circadian auto-dimming that follows a configurable daily curve (new).

### 4.3 RGBW Lighting

- ON/OFF, brightness control, RGB colour selection, white-channel control.

- Colour presets and scene-based colour selection.

### 4.4 Tunable White

- Brightness control and colour-temperature adjustment.

- Presets: Warm, Neutral, Cool.

- Automatic colour-temperature scenes tied to time of day where programmed.

### 4.5 Lighting Scenes

Predefined Scenes Example — Relax — reduce selected lights, set warm colour temperature, activate selected decorative lighting. Example — Movie — turn off unnecessary lighting, dim entertainment-area lights, activate audio. Example — Dinner — set dining lights to a predefined level, reduce surrounding lighting. Example — Night — switch off non-essential lights, maintain selected night/circulation lighting.

# 5. HVAC & Climate Control

### 5.1 Display

- Current room temperature, target temperature, HVAC operating mode.

- Fan status, heating status, cooling status, window status.

### 5.2 Controls

- Heating ON/OFF, cooling ON/OFF, fan ON/OFF.

- Temperature setpoint, operating mode, and schedule.

- Per-zone setback temperatures for Away and Night modes.

### 5.3 Window Interlock

- When the app receives an OPEN signal from a window sensor, it displays the window as open and generates a notification where configured.

- HVAC may reduce or inhibit cooling/heating for that zone while the window is open.

- The system resumes normal operation automatically once the window is closed.

- Interlock logic (which zones, delay before action, override permissions) is fully configurable.

### 5.4 Indoor Air Quality Integration (new)

- Continuous CO₂, VOC, humidity, and PM2.5 telemetry surfaced alongside temperature.

- Automatic increase of ventilation/filtration when the air-quality index crosses a configured threshold.

- Historical IAQ trend charts, matching the format used for energy history.

# 6. Security System & Video Telemetry

### 6.1 Core Security Functions

- Arm system, disarm system, Away mode, Home mode, Night mode.

- View active alarms, sensor status, and event history.

### 6.2 Sensors

- Motion detectors and presence sensors used for security logic.

- Door status, window status, smoke/CO status, water-leak status, alarm status.

### 6.3 Alarm Indication

Active Alarm Example — WATER LEAK DETECTED — Location: Kitchen — Time: 21:34

The event remains visible until acknowledged or cleared according to the configured alarm logic.

### 6.4 Live Video (new pillar, merged from six-pillar spec)

- Live WebRTC streaming with sub-200 ms glass-to-glass latency and two-way audio.

- Local Pan-Tilt-Zoom (PTZ) controls for supported cameras.

- Local NVR storage plus optional cloud clip backup, with a scrubbable timeline.

- Event-triggered clips for motion, vehicle detection, and facial recognition, with on-device pre-filtering to reduce false alerts.

- Privacy indicator shown in-app whenever a camera or microphone is actively streaming (new).

- Facial-recognition data processed and stored locally by default, with cloud processing as an explicit opt-in (new).

### 6.5 Emergency Mode

- Single-tap panic/alarm state that locks down perimeter doors, turns on indoor cameras, flashes exterior lights red, and dispatches panic notifications.

- Configurable escalation: silent notification to household first, then to a monitoring service or emergency contacts after a timeout (new).

# 7. Access Control

### 7.1 Functions

- Lock/unlock entrance, NFC access, PIN/code access.

- User access permissions, access event history, intercom communication, entrance notification.

### 7.2 Temporary Guest Passkeys (new, merged)

- Time-limited PIN codes or app tokens generated for maintenance workers, cleaners, or visitors.

- Configurable validity window (one-time, date range, or recurring weekly window — e.g. cleaner every Tuesday 9–11 AM).

- Automatic revocation at expiry and a log entry recorded for every use.

### 7.3 User Permission Levels

| User | Access |
| --- | --- |
| Homeowner | Full control |
| Family | Normal control |
| Guest | Limited control, time-boxed if issued a passkey |
| Administrator | Configuration |

# 8. Audio / Multi-Room Music

### 8.1 Functions

- Play, pause, stop, previous/next.

- Volume control, mute, source selection, room selection.

### 8.2 Grouping

- Group rooms for synchronized playback, or run separate playback per room.

Multi-Room Examples Example — Living Room + Dining + Kitchen grouped → play the same source across all three. Example — Living Room → Music A, while Bedroom independently plays Music B.

### 8.3 Voice & Streaming Integration (new)

- Native support for common streaming-service accounts (single sign-in, shared across rooms).

- Voice-assistant handoff so a song started by voice can be transferred to a chosen room group from the app.

# 9. Energy & Health Analytics

### 9.1 Real-Time Data

- Voltage, current, power, power factor (where available), frequency (where available), total power consumption.

### 9.2 Historical Data & Graphs

- Hourly, daily, weekly, monthly, and yearly consumption history.

- Graphical presentation of current consumption, energy trend, peak demand, and historical consumption.

- Power disaggregation: visual breakdown of consumption by room or appliance category (merged).

### 9.3 Tariff & Cost Optimization (merged)

- Automatic scheduling of high-draw loads (EV charging, heat pumps, pool pumps) to match off-peak utility pricing.

- Live cost-per-hour estimate alongside the power graph where tariff data is available.

- Solar generation and home-battery state-of-charge tracking, including grid-export vs self-consumption split (new).

### 9.4 Energy Alerts

- Notify when consumption exceeds a predefined threshold, when unusual consumption occurs, or when demand reaches a configured peak.

### 9.5 Indoor Health Analytics (new)

- Rolling air-quality score combining CO₂, VOC, humidity, and PM2.5 into a single daily home-health index.

- Correlation view showing occupancy patterns against air quality and energy use, to help identify wasteful schedules.

# 10. Advanced Automation Engine

Automation is the core intelligence of the application, extended here from simple trigger/action rules to multi-variable logic, geofencing, and solar-aware scheduling.

### 10.1 Rule-Based Triggers

- Trigger → Condition → Action model, extended with multi-variable AND/OR logic chains.

- Rules can combine device state, time, sensor readings, and presence in a single condition set.

Automation Examples Example — Presence detected AND light level below configured level → turn corridor light ON. Example — No presence AND 10-minute timeout → turn light OFF. Example — Window opened → modify HVAC operation for that zone. Example — Water detected → generate alarm and send notification. Example — 11:00 PM → activate Night scene. Example — 10 PM AND door unlocked AND motion absent → lock door and set thermostat setback (merged, multi-variable).

### 10.2 Geofencing & Proximity

- Dynamic GPS radius per household member to trigger arrival/departure scenes automatically.

- Optional Ultra-Wideband (UWB) precision for last-mile "approaching the front door" triggers.

- Per-user geofence opt-out for privacy.

### 10.3 Astronomical Scheduling

- Solar-tracking automation for shading and exterior lighting tied to real-time sunrise, sunset, and solar elevation.

- Offset rules (e.g., "30 minutes after sunset") layered on top of the solar calendar.

### 10.4 Predictive & Adaptive Automation (new)

- On-device learning of recurring occupancy and usage patterns to suggest new automations to the user.

- Anomaly flagging (e.g., a door unlocked at an unusual hour) surfaced as a suggested review, not an automatic action.

- Automation simulation/"dry run" mode that shows what a rule would have done over the past 7 days before it is enabled.

### 10.5 Voice & AI Assistant Integration (new)

- Native support for Alexa, Google Assistant, and Siri (via Matter/HomeKit) for voice control of rooms, scenes, and devices.

- In-app conversational assistant for natural-language automation authoring (e.g., "dim the living room after sunset").

- Assistant responses for status queries ("is the back door locked?") sourced from live state, never cached data older than the real-time sync interval.

# 11. Scenes & Operating Modes

### HOME

- Normal operating condition.

### AWAY

- Switch off selected lighting.

- Reduce HVAC operation.

- Activate security functions.

- Maintain essential systems.

### NIGHT

- Switch off non-essential lighting.

- Activate selected low-level lighting.

- Maintain security monitoring.

- Adjust HVAC according to night settings.

### VACATION

- Energy-saving operation.

- Security monitoring.

- Optional presence simulation (randomized lighting to suggest occupancy).

- Remote notifications.

# 12. Notifications

### 12.1 Critical

- Intrusion/alarm, smoke/CO alarm, water leakage, major system fault.

### 12.2 Warning

- Window left open, device offline, unusual energy consumption, communication failure.

### 12.3 Information

- Scene activated, home mode changed, device state changed.

Each notification should include: Event + Location + Time + Current Status.

### 12.4 Delivery Channels (new)

- Push notification, in-app banner, and optional SMS/email fallback for critical alerts when the app is unreachable.

- Per-category notification muting (e.g., mute "device offline" warnings during a scheduled firmware rollout).

# 13. Water Leak Detection

### 13.1 Behaviour

- Sensor changes to alarm state; the app receives the event and identifies the affected location.

- User receives a notification; alarm/siren activates where configured; the event is logged.

- Alarm remains active until reset or cleared.

Active Leak Example — WATER LEAK DETECTED — Location: Kitchen — Status: ACTIVE

### 13.2 Automated Mitigation (new)

- Optional automatic main water-shutoff valve trigger on confirmed leak, where a compatible valve is installed.

- Configurable "confirm before shutoff" grace period to avoid false-positive shutoffs.

# 14. Weather Station Integration

### 14.1 Data Points

- Outdoor temperature, humidity, wind speed, wind direction, rainfall, atmospheric pressure, weather trends.

Weather information may also act as an automation input — for example, closing exterior shades automatically ahead of forecast high wind, or pausing irrigation ahead of forecast rain (new).

# 15. Scheduling

### 15.1 Schedulable Items

- Lighting, HVAC, scenes, audio, and other controllable loads.

Weekday Schedule Example — 07:00 → Morning Scene Example — 18:00 → Evening Scene Example — 22:30 → Night Scene

### 15.2 Schedule Types

- Daily, weekly, and one-time schedules.

- Multiple concurrent schedules per device or scene.

- Enable/disable toggle per schedule without deleting it.

- Holiday/exception calendar that overrides the normal weekly pattern (new).

# 16. Device Management

### 16.1 Device List

- Device name, room, device type, online/offline status, current state, communication status.

### 16.2 Extended Detail (where supported)

- Fault condition, last communication timestamp, battery status, device address/identifier.

- Firmware version and pending-update indicator, linked to the OTA pipeline in Section 1.3 (merged).

# 17. System Diagnostics

### 17.1 Installer/Administrator View

- Controller status, network status, device communication, sensor status, input/output status.

- Fault logs, alarm logs, system events, communication errors.

This section should normally be restricted to authorized users, aligned with the "technician mode" introduced in Section 1.4.

# 18. Remote Access

### 18.1 Capabilities

- Check house status, lights, windows, security, and HVAC status.

- View energy consumption, activate scenes, and receive alarm notifications remotely.

Remote control is subject to secure authentication and the system's connectivity/service architecture.

### 18.2 Offline LAN Mode (merged)

- Uninterrupted local control executing routines over the local network (Thread / local mDNS) during wide-area network outages.

- Automatic queuing and replay of missed cloud-dependent actions (e.g., remote notifications) once connectivity returns.

# 19. User Management & Role-Based Access Control

### 19.1 Account Functions

- User registration, login, password/PIN, biometric authentication where supported.

- User roles, permission management, user removal, guest access.

### 19.2 Example Permission Levels

| Role | Description |
| --- | --- |
| Owner | Full system control. |
| Family | Room and scene control. |
| Guest | Restricted room/device control, optionally time-boxed. |
| Installer | Technical configuration and diagnostics. |

### 19.3 Granular RBAC (merged)

- Feature-level permission flags (e.g., a Family member can control lighting but not disarm security).

- Time-window restrictions per role (e.g., a Guest role that only functions between 8 AM and 8 PM).

# 20. Event History

| Time | Event | Location |
| --- | --- | --- |
| 07:02 | Morning Scene Activated | House |
| 07:15 | Presence Detected | Kitchen |
| 12:20 | Window Opened | Bedroom |
| 18:30 | Evening Scene Activated | House |
| 21:45 | Water Sensor Normal | Kitchen |

The user should be able to filter events by date, room, event type, and severity.

### 20.1 Retention & Export (new)

- Configurable local retention period, with optional export to CSV for longer-term recordkeeping.

- Read-only audit export for insurance or compliance purposes, scoped to security and access events.

# 21. App Security & Data Protection

### 21.1 Baseline Controls

- Secure login, strong authentication, role-based access, session timeout.

- Encrypted communication, secure remote access, audit/event logging, permission-controlled device access.

Critical operations should require appropriate re-authentication (e.g., disarming security, adding a new user, or issuing a guest passkey).

### 21.2 Extended Data-Protection Requirements (new)

- End-to-end encryption for video streams and stored clips, with keys held on-device where feasible.

- Data minimization: raw sensor streams processed locally where possible; only derived events sent to the cloud.

- Regional data-residency options for cloud-stored history and clips.

- Automatic session revocation on suspected compromise (e.g., repeated failed PIN attempts at a smart lock).

# 22. Backup, Resilience & Disaster Recovery (new)

### 22.1 Configuration Backup

- Automatic periodic backup of scenes, automations, schedules, and user/role configuration to the cloud account.

- One-tap restore to a replacement hub after hardware failure.

### 22.2 Continuity

- Local hub battery backup with a minimum runtime target, surfaced in Diagnostics.

- Graceful degradation order during partial outages: security and life-safety functions (smoke/CO, water leak, locks) prioritized over convenience functions (lighting, audio).

# 23. Third-Party Ecosystem & Interoperability (new)

### 23.1 Standards Support

- Matter over Thread and Wi-Fi as the primary interoperability layer.

- Zigbee and Z-Wave bridging for legacy devices via a certified hub.

- Apple HomeKit, Google Home, and Amazon Alexa bridges for households running multiple ecosystems.

### 23.2 Open Integration

- IFTTT or equivalent webhook/automation bridge for niche integrations outside the core partner list.

- Documented local REST/event API (see Appendix B) for advanced users and integrators.

# 24. Accessibility (new)

### 24.1 Requirements

- Full VoiceOver (iOS) and TalkBack (Android) support across all primary control flows.

- Minimum WCAG 2.1 AA colour-contrast compliance, including in dark mode.

- Adjustable text size independent of device-wide OS settings, and support for reduced-motion mode.

- Voice-assistant control as an alternative interaction path for users with limited mobility or vision.

# 25. Non-Functional Requirements (new)

### 25.1 Performance Targets

| Metric | Target |
| --- | --- |
| Local device state sync | ≤ 100 ms, local network |
| Cloud/remote command round-trip | ≤ 1.5 s, typical broadband |
| Live video glass-to-glass latency | ≤ 200 ms, local network |
| App cold start | ≤ 2 s to interactive dashboard |
| Automation rule evaluation | ≤ 500 ms from trigger to action dispatch |

### 25.2 Scalability

- Single property supports at minimum 250 devices and 20 concurrent user sessions without degradation.

- Multi-property accounts scale independently per property; dashboard load time must not increase materially with additional properties on the account.

### 25.3 Reliability & Compliance

- Target 99.9% cloud-service availability; local control must continue functioning during cloud outages (see Section 18.2).

- Compliance alignment with applicable regional standards for electrical safety, radio equipment, and data protection (e.g., UL/ETL, CE/RED, GDPR/local equivalents), to be confirmed per deployment jurisdiction.

- Security certification alignment with Matter's device-attestation model for all Matter-commissioned devices.

# 26. Technology Stack Recommendations (new)

| Layer | Recommended Approach |
| --- | --- |
| Mobile client | Cross-platform (React Native or Flutter) with native modules for widgets/lock-screen controls |
| Local transport | Thread + Matter, with Zigbee/Z-Wave via bridge |
| Real-time sync | MQTT broker or WebSocket gateway on the local hub, mirrored to cloud |
| Cloud backend | Managed container platform, regional deployment for data-residency needs |
| Video pipeline | WebRTC with on-hub edge processing for motion/face pre-filtering |
| Automation/rules engine | Rule graph evaluated on-hub first, cloud as fallback |
| Analytics/history store | Time-series database for energy/IAQ, relational store for events/users |

# 27. Recommended Main App Menu

For the Cecil Amartey Abbey project, the recommended top-level information architecture:

### HOME

- Dashboard

### ROOMS

- Individual room controls

### SCENES

- Home / Away / Night / Movie / Relax / Entertain

### SECURITY

- Alarm / Access / Sensors / Doors / Windows / Cameras

### CLIMATE

- HVAC / Temperature / Ventilation / Air Quality

### ENERGY

- Power / Consumption / History / Tariff Optimization

### AUDIO

- Music / Rooms / Volume

### AUTOMATION

- Rules / Triggers / Schedules / Geofencing

### NOTIFICATIONS

- Alerts / Events

### SETTINGS

- Users / Devices / System / Properties / Preferences / Accessibility

# Appendix A — Starter Data Model (new)

A minimal entity set to seed detailed database design. Field lists are illustrative, not exhaustive.

| Entity | Key Fields |
| --- | --- |
| Property | property_id, name, address, timezone, owner_user_id |
| User | user_id, name, role, auth_method, property_ids[] |
| Room | room_id, property_id, floor, name, zone_ids[] |
| Device | device_id, room_id, type, protocol, firmware_version, state, last_seen |
| Scene | scene_id, property_id, name, device_actions[] |
| Automation | automation_id, property_id, trigger, conditions[], actions[], enabled |
| Event | event_id, property_id, device_id, type, severity, timestamp, location |
| EnergyReading | reading_id, property_id, device_id, timestamp, watts, source |

# Appendix B — Local REST / Event API Sketch (new)

### Representative REST Endpoints

- GET /properties/{id}/devices — list devices and current state.

- POST /devices/{id}/commands — send a command (e.g., set brightness, lock, set setpoint).

- GET /properties/{id}/scenes — list scenes; POST /scenes/{id}/activate — run a scene.

- GET /properties/{id}/events?since=&type=&severity= — filtered event history.

- POST /automations — create a rule; PATCH /automations/{id} — enable/disable/edit.

### Representative Real-Time Events (WebSocket/MQTT topics)

- home/{property_id}/device/{device_id}/state — device state change.

- home/{property_id}/alarm — security or life-safety alarm raised or cleared.

- home/{property_id}/energy/live — streaming power reading.

- home/{property_id}/automation/{id}/fired — automation execution log entry.